.

2.  Developmental work : The double layer experiment (DLX)

An experimental setup has been constructed for the study of plasma propulsion using the concept of electric double layer. An inductive RF discharge has been produced in a quartz tube by a loop antenna connected to a RF source (13.56 MHz, 1.25 kW) through a π-type matching network. The tube is kept in an axial magnetic field (500 G maximum) provided by a Helmholtz coil pair. The plasma (n~1e11 cm-3, Te~10 eV) diffuses into a larger chamber (50 cm diameter, 50 cm long) along a diverging magnetic field. Formation of an electric double layer has been observed under such situations, which can accelerate ions to high energies as a beam.

A RF-compensated Langmuir probe, an emissive probe and a four-grid retarding field ion energy analyser have also been constructed and used to find the density, electron temperature, plasma potential and energy distribution function of the ions in two dimensions.

The 2-D map of the plasma potential Vp reported earlier showed that the equipotential surfaces near the axis are U-shaped and a secondary lobe exists off-axis on either side. The minimum of the radial profile of Vp was found to lie on the most diverging magnetic field line passing through the radial edge of the source tube. We have extended this measurement to study the 2-D map of the density. The density has a peaked radial profile near the source as expected. However, the density exhibits a hollow conical structure away from the source, the maximum density being on the same magnetic field line. This peak density is found to increase slowly along the same field line and is a new observation.

A better understanding of these 2-D structures is necessary for the eventual deployment of such devices for space thruster application.

S. Chowdhury, M. S. Janaki, Abhijit Ghosh, S. K. Saha*, A. K. Hui* and S. Raychaudhuri*

* external collaborator